VEHICLE DASHBOARD USING CAN PROTOCOL

DASHBOARD TRANSMITTER CODE

/* ========================= transmit.c ========================= */

/* Main Dashboard Node */

#include <LPC21xx.H>

#include "lcddisplay.h"

#include "hadd.h"

#define SW1 10

#define SW2 11

#define M1 16

#define M2 17

#define LED 19          /* active low */

int main()

{

CAN2_MSG m1,m2;

unsigned int v;

LCD_INIT();

can2_init();

/* OUTPUT PINS */

IODIR0 |= (1<<M1) | (1<<M2) | (1<<LED);

/* SWITCH INPUT PINS */

IODIR0 &= ~(1<<SW1);

IODIR0 &= ~(1<<SW2);

/* INITIAL STATES */

IOCLR0 = (1<<M1) | (1<<M2);

IOSET0 = (1<<LED);

LCD_COMMAND(0x80);

LCD_STR("VEHICLE");

LCD_COMMAND(0xC0);

LCD_STR("DASHBOARD");

delay_second(2);

LCD_COMMAND(0x01);

while(1)

{

/* ---------- TEMPERATURE ---------- */

if(((IOPIN0>>SW1)&1)==0)

{

delay_millisecond(200);

m1.id    = 0x101;

m1.rtr   = 1;

m1.dlc   = 1;

m1.byteA = 0;

m1.byteB = 0;

can2_tx(m1);

can2_rx(&m2);

if((m2.id==0x101) && (m2.rtr==0))

{

LCD_COMMAND(0x01);

LCD_COMMAND(0x80);

LCD_STR("TEMP:");

LCD_INTEGER(m2.byteA);

LCD_DATA(0xDF);

LCD_DATA('C');

if(m2.byteA > 25)

{

IOSET0 = (1<<M1);

IOCLR0 = (1<<M2);

}

else

{

IOCLR0 = (1<<M1);

IOCLR0 = (1<<M2);

}

}

while(((IOPIN0>>SW1)&1)==0);

}

/* ---------- FUEL ---------- */

if(((IOPIN0>>SW2)&1)==0)

{

delay_millisecond(200);

m1.id    = 0x102;

m1.rtr   = 1;

m1.dlc   = 1;

m1.byteA = 0;

m1.byteB = 0;

can2_tx(m1);

can2_rx(&m2);

if((m2.id==0x102) && (m2.rtr==0))

{

v = m2.byteA;

LCD_COMMAND(0x01);

LCD_COMMAND(0x80);

if(v < 10)

{

LCD_STR("FUEL:LOW");

IOCLR0 = (1<<LED);

}

else if(v < 25)

{

LCD_STR("FUEL:MEDIUM");

IOSET0 = (1<<LED);

}

else

{

LCD_STR("FUEL:HIGH");

IOSET0 = (1<<LED);

}

LCD_COMMAND(0xC0);

LCD_STR("VOLT:");

LCD_INTEGER(v/10);

LCD_DATA('.');

LCD_INTEGER(v%10);

LCD_DATA('V');

}

while(((IOPIN0>>SW2)&1)==0);

}

}

}

TEMPERATURE RECEIVER CODE

/* ========================= receiver.c ========================= */

/* Temperature Node */

#include <LPC21xx.H>

#include "can_d.h"

#define CS        7

#define MSTR_BIT  5

#define Mode_3    0x18

#define SPIF_BIT  7

CAN2_MSG m1,m2;

unsigned char SPI0(unsigned char data);

float Read_ADC_MCP3204(unsigned char ch);

/* SPI INIT */

void Init_SPI0(void)

{

PINSEL0 |= 0x00001500;

S0SPCCR = 60;

S0SPCR  = (1<<MSTR_BIT) | Mode_3;

IODIR0 |= (1<<CS);

IOSET0  = (1<<CS);

}

/* SPI DATA */

unsigned char SPI0(unsigned char data)

{

unsigned char stat;

stat = S0SPSR;

S0SPDR = data;

while((S0SPSR & (1<<SPIF_BIT)) == 0);

return S0SPDR;

}

/* ADC READ */

float Read_ADC_MCP3204(unsigned char ch)

{

unsigned char h,l;

unsigned int val = 0;

IOCLR0 = (1<<CS);

SPI0(0x06);

h = SPI0(ch<<6);

l = SPI0(0x00);

IOSET0 = (1<<CS);

val = ((h & 0x0F)<<8) | l;

return ((val * 3.3) / 4096);

}

/* MAIN */

int main()

{

float volt;

int temp;

can2_init();

Init_SPI0();

while(1)

{

volt = Read_ADC_MCP3204(0);

temp = volt * 100;

can2_rx(&m1);

if((m1.id==0x101) && (m1.rtr==1))

{

m2.id    = 0x101;

m2.rtr   = 0;

m2.dlc   = 1;

m2.byteA = temp;

m2.byteB = 0;

can2_tx(m2);

}

}

}

FUEL RECEIVER CODE

/* ---------- receiver2.c ---------- */

/* Fuel Node */

#include <LPC21xx.H>

#include "can_d.h"

#define CS        7

#define MSTR_BIT  5

#define Mode_3    0x18

#define SPIF_BIT  7

CAN2_MSG m1,m2;

unsigned char SPI0(unsigned char data);

float Read_ADC_MCP3204(unsigned char ch);

/* SPI INIT */

void Init_SPI0(void)

{

PINSEL0 |= 0x00001500;

S0SPCCR = 60;

S0SPCR  = (1<<MSTR_BIT) | Mode_3;

IODIR0 |= (1<<CS);

IOSET0  = (1<<CS);

}

/* SPI DATA */

unsigned char SPI0(unsigned char data)

{

unsigned char stat;

stat = S0SPSR;

S0SPDR = data;

while((S0SPSR & (1<<SPIF_BIT)) == 0);

return S0SPDR;

}

/* ADC READ */

float Read_ADC_MCP3204(unsigned char ch)

{

unsigned char h,l;

unsigned int val = 0;

IOCLR0 = (1<<CS);

SPI0(0x06);

h = SPI0(ch<<6);

l = SPI0(0x00);

IOSET0 = (1<<CS);

val = ((h & 0x0F)<<8) | l;

return ((val * 3.3) / 4096.0);

}

/* MAIN */

int main()

{

float volt;

can2_init();

Init_SPI0();

while(1)

{

volt = Read_ADC_MCP3204(0);

can2_rx(&m1);

if((m1.id==0x102) && (m1.rtr==1))

{

m2.id    = 0x102;

m2.rtr   = 0;

m2.dlc   = 1;

m2.byteA = (unsigned int)(volt * 10);

m2.byteB = 0;

can2_tx(m2);

}

}

}

LCD.H

/* lcd.h*/

#include <LPC21xx.H>

#include "delay.h"

#define LCD_D (0xff<<0)

#define RS (1<<8)

#define E  (1<<9)

void LCD_INIT(void);

void LCD_COMMAND(unsigned char);

void LCD_DATA(unsigned char);

void LCD_INTEGER(int);

void LCD_STR(unsigned char *);

void LCD_SCROLL(unsigned char *);

void LCD_FLOAT(float);

void LCD_INIT(void)

{

IODIR0 |= LCD_D|RS|E;

LCD_COMMAND(0x01);

LCD_COMMAND(0x02);

LCD_COMMAND(0x0C);

LCD_COMMAND(0x38);

}

void LCD_COMMAND(unsigned char cmd)

{

IOCLR0 = LCD_D;

IOSET0 = cmd;

IOCLR0 = RS;

IOSET0 = E;

delay_millisecond(2);

IOCLR0 = E;

}

void LCD_DATA(unsigned char d)

{

IOCLR0 = LCD_D;

IOSET0 = d;

IOSET0 = RS;

IOSET0 = E;

delay_millisecond(2);

IOCLR0 = E;

}

void LCD_INTEGER(int n)

{

unsigned char num[20];

int i=0;

if(n==0)

{

LCD_DATA('0');

return;

}

if(n<0)

{

LCD_DATA('-');

n=-n;

}

while(n>0)

{

num[i++]=n%10;

n=n/10;

}

for(i=i-1;i>=0;i--)

{

LCD_DATA(num[i]+48);

}

}

void LCD_FLOAT(float n)

{

int temp=(int)n;

LCD_INTEGER(temp);

LCD_DATA('.');

LCD_INTEGER((n-temp)*100);

}

void LCD_STR(unsigned char *d)

{

while(*d!='\0')

{

LCD_DATA(*d++);

}

}

void LCD_SCROLL(unsigned char *s)

{

int len=0,i,j;

while(s[len]!='\0')

len++;

for(i=0;i<=len;i++)

{

LCD_COMMAND(0x80);

for(j=0;j<16;j++)

{

if((i+j)<len)

LCD_DATA(s[i+j]);

else

LCD_DATA(' ');

}

delay_millisecond(300);

}

}

DELAY.H

/*delay.h*/

#include <LPC21xx.H>

void delay_millisecond(unsigned int millisecond)

{

T0PR=15000-1;

T0TCR=0x01;

while(T0TC<millisecond);

T0TCR=0x03;

T0TCR=0x00;

}

void delay_second(unsigned int second)

{

T0PR=15000000-1;

T0TCR=0x01;

while(T0TC<second);

T0TCR=0x03;

T0TCR=0x00;

}

void delay_microsecond(unsigned int microsecond)

{

T0PR=15-1;

T0TCR=0x01;

while(T0TC<microsecond);

T0TCR=0x03;

T0TCR=0x00;

}

CAN.H

/* can_d.h */

#include <LPC21xx.H>

typedef struct CAN2

{

unsigned int id;

unsigned int rtr;

unsigned int dlc;

unsigned int byteA;

unsigned int byteB;

}CAN2_MSG;

/* function prototypes */

void can2_init(void);

void can2_tx(CAN2_MSG m1);

void can2_rx(CAN2_MSG *m1);

/* ---------------- CAN INIT ---------------- */

void can2_init(void)

{

PINSEL1 |= 0x00014000;     /* P0.23 = RD2 , P0.24 = TD2 */

VPBDIV = 1;               /* PCLK = CCLK */

C2MOD = 0x01;            /* reset mode */

AFMR  = 0x02;            /* accept all */

C2BTR = 0x001C001D;      /* 125 kbps */

C2MOD = 0x00;            /* normal mode */

}

/* ---------------- CAN TX ---------------- */

void can2_tx(CAN2_MSG m1)

{

C2TID1 = m1.id;

C2TFI1 = (m1.dlc << 16);

if(m1.rtr == 0)

{

C2TFI1 &= ~(1<<30);   /* data frame */

C2TDA1 = m1.byteA;

C2TDB1 = m1.byteB;

}

else

{

C2TFI1 |= (1<<30);    /* remote frame */

}

C2CMR = (1<<0) | (1<<5);

while((C2GSR & (1<<3)) == 0);

}

/* ---------------- CAN RX ---------------- */

void can2_rx(CAN2_MSG *m1)

{

while((C2GSR & 1) == 0);

m1->id  = C2RID;

m1->dlc = (C2RFS >> 16) & 0x0F;

m1->rtr = (C2RFS >> 30) & 0x01;

if(m1->rtr == 0)

{

m1->byteA = C2RDA;

m1->byteB = C2RDB;

}

C2CMR = (1<<2);          /* release buffer */

}